# One Stop Solution for DFT Register

# Modelling in UVM

# Rui Huang

Advanced Micro Devices, Inc., Rui.Huang@amd.com

No.2, Science Institute South Rd.

# Haidian District, Beijing, China 100190

Abstract-The DFT (Design For Test) design is becoming more and more complex to satisfy test requirements for ultra-large-scale SoC (System on Chip). From the perspective of test access method, nowadays IEEE 1149.1 protocol is usually adopted along with IEEE 1687 and 1500 protocols, which enables easy and modular integration of DFT IP (Intellectual Property) into SoC. However, this approach makes the DFT test access network complex and it needs a serial of complex shift operations to access a TDR (Test Data Register). It will be beneficial if abstracting DFT TDR access in RAL (Register Abstract Level), so that test writers can focus on test sequences and tests can easily migrate from block level to system level. This paper introduces a layered structure to model DFT TDRs and its access network, which is universal for different projects. Also this paper introduces a new way to model ultra-long length registers in UVM that requires smaller memory space in simulation than the current UVM RAL solution.

# I.   Introduction

# To model DFT TDR in UVM, the first challenge is how to model ultra-long length TDRs. In particular, some DFT TDRs’ length, compared to the functional registers of a SoC, can be as long as thousands of bits. At some situations such as MBIST (Memory Built-In Self-Test) dumping or scan dumping, the TDRs’ length can be even longer. The UVM RAL mainly targets at functional registers and subjects to its limitation when modelling the ultra-long length DFT TDR.

# If we want to model DFT TDRs of a system using the current UVM RAL solution, we need to override the two macros UVM_REG_DATA_WIDTH and UVM_REG_ADDR_WIDTH to the value identical to the length of the longest DFT TDR in the system. The width of the two basic data types in UVM RAL, namely uvm_reg_addr_t and uvm_reg_data_t, is decided by the abovementioned UVM_REG_ADDR_WIDTH and UVM_REG_DATA _WIDTH macros, respectively. These two data types are constructed, and are used almost everywhere in the RAL-related components and objects. Furthermore, when constructing a TDR, every field of that TDR is modelled as uvm_reg_field that also profligately uses the uvm_reg_data_t type to store the field value, although every field of the DFT TDR is not long. As we can see the waste of storage in simulation is obvious.

This paper presents a new way to model ultra-long length DFT TDR using UVM, with a minor amount of work needed to override the methods of the base classes of UVM RAL.

In a complex DFT test access network, different protocol TDRs are hierarchically located in a network that is connected via IEEE 1687. To access a TDR, one or more levels 1687 SIBs (Segment Insertion Bit) have to be opened, and the length of DR (Data Register) chain varies with SIB values, which results in the second challenge: how to get the necessary information to convert the abstracted generic TDR access operation into a series of IEEE 1149.1 shift operations (hereinafter called JTAG operations) – and ideally this solution can be applied to different projects. Recently the author [1] introduced a novel method to model DFT TDR access network, where functional equivalent elements are created as the DUT (Design Under Test) and information required in converting an abstracted TDR access to JTAG operations is encoded into the TDR’s address.

The disadvantage of [1] is that, when a TDR is accessed, its controlling SIBs are opened, desired value is shifted to the TDR, and then the SIBs are closed to their default values for the convenience of the next TDR access. However, in real test scenarios, it turns out that TDRs controlled by identical SIBs are accessed probably in tandem, which means that we can save some unnecessary SIBs opening and closing JTAG operations, and thus some test time in ATE test.

This paper tries to improve the efficiency of converting a generic TDR access operation to JTAG operations introduced by [1], it saves shift cycles by means of monitoring the status of current network SIBs and analyzing the TDR to be accessed so as to open and close the SIBs smartly.

## Structure of This Paper

This paper introduces a layered structure to model DFT TDR. As shown in Figure 1, the DFT TDR layer is divided into two layers. In the Register Layer One, the generic DFT TDR access is converted into the generic dft_reg_transaction, and the dft_reg_monitor writes the observed dft_reg_transaction to the dft_reg_predictor.  In the Register Layer Two, the generic dft_reg_transaction is converted into several jtag_transactions, and the dft_tdr_network returns the dft_reg_transaction by observing jtag_transactions written by the jtag_monitor. In the Transaction Layer, the jtag_transactions are passed to the jtag_driver to toggle JTAG interface.

Accordingly, this paper is divided into three sections. The first section is about how to model ultra-long length TDR in the Register Layer One, while the second section explains how to model DFT TDR access network in the Register Layer Two. The third part is results and discussion.

In each of the first and second parts, a general overview will be provided firstly, and then the detailed implementation will be elaborated with reference to examples.

# II.   Ultra-Long Length TDR Modelling in Register Layer One

## Idea Overview

Regarding to UVM register modelling on ultra-long length TDRs, the author analyzed the methods being called during a register write and read process in UVM RAL, and found that the major limitations lies in the following two facts:

- The register access methods of the uvm_reg class uses uvm_reg_data_t as the routine argument or return type for generic register access operations such as uvm_reg::write() and uvm_reg::read().

- Most methods in UVM RAL suppose that the dynamic array size of uvm_reg_item.value is one.

Figure 1. DFT TDR modelling block diagram.

To eliminate these limitations, the following steps can be taken:

- As shown in Figure 2, the dft_reg class is defined as the base class when construing DFT TDRs. A set of register access methods with “dft” as prefix are added to replace the corresponding ones in uvm_reg class. This set of methods uses dft_reg_data_t, which is a queue of bit type, as routine argument and return type instead of uvm_reg_data_t. In this way, no matter how long the TDR length is, the queue size can dynamically fit to the TDR’s length.

- Modify the methods supposing the dynamic array size of uvm_reg_item.value to be one, so that they construct uvm_reg_item.value array according to the bit length of the data being processed, or they fetch the data in uvm_reg_item.value array by checking its size first.

Figure 3 illustrates how a generic DFT TDR write access is converted to a generic dft_reg_transaction in the view of routine arguments passing.

- The value to be written in a TDR is passed to the dft_reg::dft_write() by the value_q argument of dft_reg_data_t type, instead of by the uvm_reg::write() value argument of uvm_reg_data_t type.

- The dft_reg::dft_set() converts value_q data into several segments of uvm_reg_data_t type, which are filled to uvm_reg_field::set().

- The dft_reg::do_write() creates a uvm_reg_item object to store the value_q data by re-constructing the dynamic array (uvm_reg_item.value) with desired size.

- The dft_reg::do_write() passes the written data to dft_reg_map::do_write() by argument rw of uvm_reg_item type.

- The dft_reg_map::do_bus_write() converts the written data stored in the dynamic array to several uvm_reg_bus_op packages, which are passed to the dft_reg_adapter.

By using the MSB (Most Significant Bit) of the address (encoded in Figure 4) as a flag bit that indicates the last package of the written data, the dft_reg_adapter::reg2bus() knows the time when all the written data are collected and to return the complete dft_reg_transaction that the dft_reg_map::do_bus_write() is going to send to the dft_reg_sequencer.

- The dft_reg_predict::write() calls dft_reg_adapter::bus2reg() for several times until the MSB of the uvm_reg_bus_op.addr is set by the dft_reg_adapter::bus2reg()  to indicate that the last package data has been converted.

- The dft_reg_predict::write() creates a uvm_reg_item object and stores the uvm_reg_bus_op.data to uvm_reg_item.value that is passed to dft_reg::do_predict().

- The dft_reg::do_predict() concatenates all the data in uvm_reg_item.value to dft_reg_data_t type, disassembles it into several uvm_reg_data_t type according to the TDR’s field width, and then passes them to uvm_reg_field::do_predict().

Figure 2. Class extension diagram.

Figure 3. Routine argument passing process of write operation.

## dft_reg Class Implementation

As shown in Figure 2, the methods with “dft_” prefix are newly added (shown in green) for DFT TDR access. To implement these newly methods, we can copy the corresponding ones in the uvm_reg class and modify the input argument (or return type) to dft_reg_data_t instead of uvm_reg_data_t. Then the data type conversion is added when it is needed. Because the  uvm_reg::do_predict() supposes the uvm_reg_item.value array size to be one, we need to use dft_reg::do_predict() to override it.

## dft_reg_block Class Implementation

The dft_reg_block class is extended from the uvm_reg_block shown in Figure 2. In Figure 2, the green methods are newly added, and the yellow methods need to be overridden. Because many methods use local variables in uvm_reg_block class which cannot be seen by extended classes, we copy all codes in uvm_reg_block to dft_reg _block class and do the following changes.

- Remove the fatal error check that uvm_reg_block::max_size should not be larger than UVM_REG_DATA_WIDTH in lock_model() function. This check is invalid for DFT TDR, because DFT TDR is configured through serial JTAG bus.

- Enhance Xinit_address_mapsX() function to support dft_reg_map type.

- Add create_dft_map() function to return dft_reg_map type register map.

## dft_reg_map Class Implementation

The dft_reg_map class is extended from the uvm_reg_map shown in Figure 2. Similar to the uvm_reg_block, many methods of the uvm_reg_map use local variables, so we copy all codes in uvm_reg_map to dft_reg_map class and do the following changes.

- Modify the do_bus_write() method to convert each element in uvm_reg_item.value to uvm_bus_reg_op.data and set the MSB of the last uvm_bus_reg_op.addr to one so as to indicate that all the written data have been transferred as shown in black arrow in Figure 4. Then dft_reg_adpater::reg2bus() returns a complete generic dft_reg_transaction to do_bus_write().

Figure 4. Data conversion process example.

- Modify get_physical_adresses() to only returning signal address no matter how long the TDR width is.

- Modify top_map to dft_reg_map type instead of uvm_reg_map type in Xinit_address_mapX(), m_set_reg _offset() and m_set_mem_offset().

- Modify add_parent_map() to supporting dft_reg_map type.

- Modify local variable m_parent  to dft_reg_block type, and modify configure() function accordingly.

## dft_reg_predictor Class Implementation

The dft_reg_predictor class is extended from uvm_reg_predictor. Add dft_map variable of dft_reg_map type and modify write() task to let it call dft_reg_adpater::bus2reg() several times until it see the MSB of the uvm_reg_bus _op.addr is set, which indicates that the observed dft_reg_transaction has been converted to several uvm_reg _bus_op packages, as show in red arrow in Figure 4. Then the write() task create a uvm_reg_item object to store the returned packages and pass the object to dft_reg::do_predictor().

# III.   DFT Test Access Network Modelling

## Idea Overview

Figure 5 is an example for DFT test access network. In Figure 6, a SIB  is modelled as sib_node, and a D flip-flop is modelled as reg_node. The out_update () method models the active clock edge that triggers the shift register bit during shift operation, while the value_update () method models the active clock edge that triggers the update register bit during the update operation. By using the sib_node and reg_node we can constructs the elements of the test access network. The possible paths from TDI (Test Data Input) to TDO (Test Data Output) can be described by System Verilog conditional statements. As such, a functional equivalent dft_tdr_network is obtained.

## dft_reg_transaction and jtag_transaction definition

Figure 7 shows the properties of the dft_reg_transaction and the jtag_transaction class.

In the dft_reg_transaction, the read_not_write indicates whether the transaction is a read or a write operation.

The addr is the TDR’s encoded address, as shown in Figure 8.

The wr_data_q stores the data to be written.

The rd_data_q stores data returning by the dft_reg_monitor.

The reg_length indicates the TDR’s length.

The extension is used to send side information to the dft_reg_adapter.

In the jtag_transaction, the o_ir stores the TDR’s OPCODE (OPeration CODE) and o_ir_length is its size. The o_dr stores the data being written to the TDR and the o_dr_length is its size.

The tdo_dr_queue, tdo_ir_queue, tdi_dr_queue, and tdi_ir_queue store the data during shifting IR or shifting DR state monitored by the jtag_monitor.

Figure 5. DFT test access network.

Figure 6. DFT test access network elements modelling.

Figure7. dft_reg_transaction and jtag_transaction properties.

Figure 8. TDR address encoding.

The chk_ir_tdo and chk_dr_tdo are flags to inform the jtag_driver whether to check TDO cycle-by-cycle during shifting IR (Instruction Register) or shifting DR state. The exp_tdo_dr_queue is the golden data expecting the DUT TDO output during shifting DR state.

The exp_tdo_dr_mask_queue indicates which bit in exp_tdo_dr_queue needs not to be checked.

The exp_tdo_ir_queue is the golden data expecting the DUT TDO output during shifting IR state.

## DFT TDR Encode

A DFT TDR’s address is encoded in Figure 8. It composes of three segments, the first segment is the reserved flag bit for the dft_reg_map and the  dft_reg_predictor that communicate with the dft_reg_adapter as mentioned in ultra-long length TDR modelling section.  The second segment is the TDR’s OPCODE, and the third segment is the TDR’s location information in the test access network. In Figure 5, WIR1’s OPCODE is 8’hFE and is controlled by LEVEL0_SIB1, so its address is encoded as 13’h0FE2.  Similarly, the WIR2’s OPCODE is 8’h36 and is controlled by LEVEL0_SIB0 and LEVEL1_SIB0, so its address is encoded as 13’h0365. For IEEE1149.1 type TDR, we can simply fill the third segment to zero.

## dft_reg_tx_to_jtag_tx_sequence Implementation

The dft_reg_tx_to_jtag_tx_sequence is a virtual sequence, which gets the dft_reg_transaction from the dft_reg _sequencer, converts it into the jtag_transactions and sends them to the jtag_sequencer. The dft_reg_tx_to_jtag_tx _sequence decodes dft_reg_transaction.addr to get the TDR’s location information in the network. Figure 9 shows how a generic dft_reg_transaction is converted to a serial of jtag_transactions. Because the dft_reg_transaction .addr[3:0] is 4’b0101, the TDR being access is controlled by LEVEL0_SIB0 and LEVEL1_SIB0, before shifting the OPCODE and written data, the LEVEL0_SIB0 and LEVEL1_SIB0 should be set first.

Figure 9. The conversion from the dft_reg_transaction to jtag_transactions

## dft_reg_network Implementation

For the test access network in Figure 5, its elements can be modelled as shown in Figure 10. The IEEE 1500 client in the test access network can be divided into two types, the first is controlled by level 0 SIBs and the second is controlled by level 1 SIBs. Because we suppose only access a WDR in a IEEE 1500 client, it is unnecessary to model every IEEE 1500 client and all the WDRs in it. It only need to model the SEL_WIR, the WIR and a WDR of each type of IEEE 1500 client. The WDR’s length is dynamic, which can be calculated by the jtag_transaction.o_dr _length and the current SIBs’ value in the dft_tdr_network.

By a serial of conditional judgments based on the possible paths between TDI and TDO in the TDR access network and the jtag_transactions observed by the jtag_monitor, the dft_tdr_network can return a generic dft_reg _transaction to the dft_reg_monitor, who passes it to the dft_reg_predictor.

In Figure 9, the step 1 to step 3 are used to open SIBs for the reason of restoring the default state of SIBs and SEL_WIRs for the convenience of access next WDR. If we want to access another TDR in the same IEEE1500 client, with the current solution, the five steps in Figure 9 are repeated. In fact the first three steps can skip if we do not close them in the step5. As mentioned above, the WDRs controlled by identical SIBs are accessed in tandem with a high possibility. If the dft_reg_tx_to_jtag_tx_sequence knew each SIB’s status in the network, then it could open and close SIBs smartly according to the current WDR being access, so as to save unnecessary SIB opening steps. In this way we can save the test time in ATE test.

To realize this, a dft_ntwk_info is defined. Figure 11 shows the properties of the dft_ntwk_info class.

A uvm_blocking_put_port is added in the dft_reg_monitor and a uvm_blocking_put_imp port is added in the dft_reg_sequencer shown in Figure 1. The dft_ntwk_info is passed from the dft_reg_monitor to the dft_reg _sequencer whenever the dft_tdr_network update SIBs. The dft_reg_tx_to_jtag_tx_sequence get current SIB’s status from dft_ntwk_info, so that it can open or close SIBs as needed.

Suppose that we want to access the WIR2 continuously for five times, it will take 840 shift cycles if using the method introduced by [1]. Now by adding a pair of uvm_blocking_put port and uvm_blocking_put_imp port to transfer the SIBs’ status, it only takes 632 shift cycles. Roughly 25% test time is saved.

# IV.   Results and Discussion

This paper introduces a layered structure to model DFT TDR in UVM. The Register Layer One and the Transaction Layer can be generally used in different projects. For Register Layer Two, in contrast, the dft_reg_network component and dft_reg_tx_to_jtag _tx_sequence are project-specific, and users need to model them according to their project’s TDR test access network architecture.

At this stage, the dft_reg_network and dft_reg_tx_to_jtag_tx_sequence are based on the assumption that only one TDR is accessed each time. However, in the next step of work, it is possible to further improve DFT TDR configure efficiency by enhancing the test access network modelling so as to support broadcast mode, which means the TDRs in different IEEE1500 clients that have the same OPCODE can be accessed simultaneously.

Table I shows five simulation results using VCS.

The simulation scenario is as follows:

A TDR is defined to have X fields and each field is one bit. Instance 200 such TDRs and write these 200 TDRs in sequence.

Figure 10. DFT access network elements mapping to defined classes.

Figure 11. dft_ntwk_info properties.

Table I

Memory Usage of Writing 200 Ultra-Long Length TDRs

|  | 512 | 1024 | 2048 | 4096 |
| --- | --- | --- | --- | --- |
| Memory Saving Solution | 2349.5 MB | 2759.5 MB | 4212.6 MB | 9263.6 MB |
| Current UVM RAL Solution | 2074.7 MB | 2344.8 MB | 2981.2 MB | 4327.2 MB |
| Memory Saving | 11.7% | 15.0% | 29.2% | 53.3% |

The row “Memory Saving Solution” shows the memory usage in simulation when using the method introduced by this paper, while the row “UVM RAL” shows the results when using the current UVM RAL solution. The percentage of memory saved is calculated. We found that 11.7% to 53.5% memory is saved when the TDR’s length ranges from 512 to 4096 bits.

Table II shows five simulation results using VCS.

The simulation scenario is as follows:

A TDR is defined to have X fields and each field is one bit. Instance 400 such TDRs and write these 400 TDRs in sequence.

The row “Memory Saving Solution” shows the memory usage in simulation when using the method introduced by this paper, while the row “UVM RAL” shows the results when using the current UVM RAL solution. The percentage of memory saved is calculated. We found that 6.8% to 60.0% memory is saved when the TDR’s length ranges from 512 to 4096 bits.

The results of Table I and Table II shows:

- The memory usage of current UVM RAL solution expands significantly when the TDR’s length larger than 2048 bits.

- The benefit of memory saving becomes increasingly obvious when the TDR’s length gets longer and the TDR numbers to be accessed increases.

Table II

Memory Usage of Writing 400 Ultra-Long Length TDRs

|  | 512 | 1024 | 2048 | 4096 |
| --- | --- | --- | --- | --- |
| Memory Saving Solution | 2335 MB | 2960.3 MB | 4066.6 MB | 6421.2 MB |
| Current UVM RAL Solution | 2504.2 MB | 3542.1 MB | 6451.5 MB | 16064 MB |
| Memory Saving | 6.8% | 16.4% | 37.0% | 60.0% |

# References

[1]	R. Huang, “A universal DFT verification environment: filling the gap between function simulation and ATE test,” DVCon 2016 (http://events.dvcon.org/events/proceedings.aspx?id=199-6).